# tProcessor-64 V2

User Guide

1	Introduction	2

2	The tProc-V2	4

2.1	Interfaces	5

2.2	Memory Organization	5

2.3	Control & Configuration Registers	7

2.4	Core	10

2.4.1	Register Organization	10

2.4.2	Special Function Registers	10

2.4.3	Random Number Generation	11

2.5	Peripherals	12

2.5.1	Time Comparator	12

2.5.2	Arithmetic Unit	12

2.5.3	Division Unit	12

2.6	Timing & Dispatcher	13

3	Python Interface	14

4	Instruction Set	15

4.1	Summary	15

4.2	Instructions options	15

4.3	ALU Operation  < -op >	16

4.4	Conditional Execution < -if() >	17

4.5	Dual task Instructions. < -wr(), -wp(), -ww >	18

4.5.1	Write Register	18

4.5.2	Write port	19

5	Assembler	20

5.1	Statements	20

5.2	Directives	20

5.3	Machine instruction Syntax	20

5.3.1	Operands	21

5.3.2	Addressing modes	21

5.3.3	Instruction description	21

5.4	Instruction Description	22

5.4.1	Configuration Instructions.	23

5.4.2	Register Instructions	23

5.4.3	Memory Instructions	24

5.4.4	Port Instructions	25

5.4.5	Branch Instructions	26

5.4.6	Block Control Instruction	27

5.5	Assembler Examples	30

5.5.1	WAIT	30

6	Architecture details	30

6.2	Pipeline Stages	30

6.3	Signals Names and Convention	31

6.4	Debugging	31

6.5	Additional Info to be added and taken into account	34

7	Signal Generator	34

# Introduction

The tProc-V2 is a custom 32-bit data, 72-bit Instruction processor designed for timed specific waveforms and data generation. Figure 1: tProcessor Block Diagram, shows a block diagram of the processor.

The processing unit is built on a t-core, with a 5-stages pipeline Von Neumann architecture, especially design for high performance timed applications. The processor with 15 instructions, is optimized to execute multiple operations in one clock cycle. Also is provided with special features including:

- Multiplication Unit (FPGA DSP) > Arithmetic unit capable of make the operation (D±A)*B±C in 2 clock cycles.

- Division unit (Custom) > Delivers the quotient and the remainder of an integer division in 32 clock cycles

- Time Comparator > Generates a Time Condition Flag when time reach a certain value.

- Pseudo Random Number Generator (LFSR) > A Configurable Linear Feedback Shift Register  is used to get a 32-bit pseudorandom number.

- Nested CALL functions.

- Debugging Capabilities

The Memory Unit has 3 Memories; The Program memory is a 72-bit memory where instructions are stored; The Data memory is a 32-bit memory where user can store and use data; The WaveParam Memory is a 128-bit memory where the parameters of the Waveform to write in the WPORT are stored.

Figure 1: tProcessor Block Diagram

This processor has 2 group of output ports: Four 32-bit Data Ports (DPORT) and sixteen Analog Wave Ports (WPORT).  The tProc-V2 is used to write specific waveforms and Data in a specific time, a 128-bit register is used to define the output waveform and 48-bit register is used to define time. The 128-bit WaveParam is composed of  6 parameters: Frequency (32-bits), Phase (32-bits), Gain (16-bits), Envelope Starting Address (16-bits), Envelope Length (16-bits), and Wave configuration (16-bits) .

A set of WaveParam (Waveform to be written) can be stored in the WaveParam memory, from the python interface, and used during the execution of the t-Proc program. User can write to a port, directly a wave from the WaveParam Memory, or can store it on a register, edited it and write it to a port, as shown in Figure 2: Wave Port Write Sources. This option gives the capability of having a big set of waves to be used and manipulated. u access either the complete WaveParam register or a single parameter. The tProc-V2 is intended to be used with the Signal Generator Block (SG). The 128-bit WaveParam output are connected to the Signal Generator.

Figure 2: Wave Port Write Sources

The tProcessor is controlled using AXI-Registers. Writing to those registers, the tProc-V2 can be controlled, and the status can be monitored.

Possible Commands from Python.

- Read / Write Program memory > Write or Read a program to/from the Program Memory

- Read / Write Data memory > Write or Read Data to/from the Data Memory

- Read / Write WaveParam memory > Write a list of Waveform to be used during the execution of the program in the WaveParam Memory.

- Reset > Reset the tProcessor. The Instruction pointer return to Zero, all the registers are cleared, and the FIFOs are flushed.

- Play > Starts / Continue with the execution of the current program.

- Stop > Stops the execution of the current program. This command stops the Processor and stop the timing (time does not run). To continue running send Play

- Pause Core > Stops the execution of the current program, but time continue running on the tProcessor.

- Freeze Time > Stops the timing, but processor continues running.

- Debugging Commands: Processor Step, Time Step, Core Step, Read Status and Debug SSignals

This core version of has no interrupts nor data stack. It has a PC-Stack that enable to nest up to 4 function CALLS.

# The tProc-V2

The tProc-V2 Processor is controlled through AXI ports. A Python interface running in the PS ( ARM Processor in the FPGA) generate and send all the signals to control the processor. All tProc-V2 instruction are 1 Cycle, with the option of executing more than one task per instruction.

- When branching, 2 clock cycles are needed to empty the pipeline.

- When a conditional instruction is executed, condition should be calculated previously, and it takes 2 clock cycles.

Figure 3: tProcessor-V2 Detailed Block Diagram

## Interfaces

The tProc-V2 provides several interfaces with different functions:

- AXI-Stream DMA Interface to load the Program, Data and WaveParam Memories with a DMA controller

- AXI-Lite Interface to Read / Write control and configuration Registers.

- Up to 8 AXI-Stream Interfaces for Readout.

- Up to Sixteen AXI-Stream Interfaces to connect SignalGeneratos (Compatible with V6)

- Up to Four 32 Bits Digital Interfaces to connect Digital Data Outpus

## Memory Organization

The processor has 3 Memories, with different interfaces for Program, Data, and Waveform to enable simultaneously instruction fetch and data load/store.

- Program Memory (PMEM): Stores the program to be executed. Is a dual port 72-bit memory (Optimized for FPGA BRAM), accesible from the tProc and from Python interface.

- Data Memory (DMEM): Stores 32-bit user data. Is a dual port 32-bit memory, accesible from the tProc and from Python interface.

- WaveParam Memory (WMEM): Stores the parameters needed to define a waveform to be written in the WPORT. The parameters are Frequency, Phase, Gain, Envelope, Length, and configuration. Is a dual port 168-bit memory, accesible from the tProc and from Python interface.

The processor has a separate Memory address Calculator, leaving the ALU free to operates, and being able to store results from the ALU in the Data Memory.

- The Processor has 4 addressing modes for the Data Memory address

  - Literal > The address is an Immediate Value

  - Register > The address is stored in a Register

  - Indexed Literal > The address is the Sum of a Register Value and a Immediate Value

  - Indexed Register > The address is the Sum of 2 registers.

- The Processor has 2 addressing modes for the WaveParam Memory address

  - Literal > The address is an Immediate Value

  - Register > The address is stored in a Register

Read ( and Write ) from ( to ) memories from the PS interface can be done in two different ways. Using a 256-bit DMA controller or using the AXI-Register Interface. The AXI-Register TPROC_CFG is used to config the process.

Figure 4: Memory Organization Block Diagram

## Control & Configuration Registers

The tProc-V2 is controlled from an AXI interface. Commands from a python Driver, running in the PS of the FPGA-PS are used to control, configure, and use the tProc-V2. 16 AXI-Registers are used to send and receive data and commands from python interface to the tProc-V2.

| ADDR | Name | Size | Description | R/W |
| --- | --- | --- | --- | --- |
| 0 | TPROC_CTRL | 32 | Control Command to tProc-V2 | R/W |
| 1 | RAND | 32 | Random Number (Read by the tProc in sreg - s1) | RO |
| 2 | TPROC_CFG | 32 | tProc Configuration | R/W |
| 3 | MEM_ADDR | 16 | Starting Address for Memory Operation | R/W |
| 4 | MEM_LEN | 16 | Length of data for Memory Operation | R/W |
| 5 | MEM_DT_I | 32 | Data for Single Write Memory Operation | R/W |
| 6 | MEM_DT_O | 32 | Data for Single Read Memory Operation | RO |
| 7 | TPROC_EXT_DT1_I | 32 | Data to the tProc (Read by the tProc in sreg – s7) | R/W |
| 8 | TPROC_EXT_DT2_I | 32 | Data to the tProc (Read by the tProc in sreg – s8) | R/W |
| 9 | PORT (LSW) | 32 | Data Port Read from the tProc (Read by the tProc in sreg – s9) | RO |
| 10 | PORT (MSW) | 32 | Data Port Read from the tProc (Read by the tProc in sreg – s10) | RO |
| 11 | TIME (USR) | 32 | Current 32-bit usr_time of the tProc (Read by the tProc in sreg - s11) | RO |
| 12 | TPROC_EXT_DT1_O | 32 | Data from the tProc (Write by the tProc in sreg - s12) | RO |
| 13 | TPROC_EXT_DT2_O | 32 | Data from the tProc (Write by the tProc in sreg - s13) | RO |
| 14 | TPROC_STATUS | 32 | tProc Status Signals | RO |
| 15 | TPROC_DEBUG | 32 | tProc Debug Signals | RO |

Table 1: Special Function Registers

To control the tProcessor an AXI-Lite interface is used. Writing the AXI-Registers, the tProc-V2 can be controlled, and the status can be monitored.

Register tproc_ctrl  is used to control or execute some tasks in the tProc from the python interface. This AXI-Register tproc_ctrl  is set to zero automatically once the task was detected.

The tasks are:

- Reset > Reset the tProcessor. The Instruction pointer return to Zero, all the registers are cleared, and the FIFOs are flushed.

- Run > Starts / Continue with the execution of the current program.

- Stop > Stops the execution of the current program. This command stops the Processor and stop the timing (time does not run). To continue running send Run.

- Pause Core > Stops the execution of the current program, but time continue running on the tProcessor.

- Freeze Time > Stops the timing, but processor continues running.

- Only for Debugging purposes Commands: Processor Step, Time Step, Core Step.

- COND set > Set the External Condition (EC) Flag.

- COND clear  > Clear the External Condition (EC) Flag .

| AXI-Register | Bit | Name | Description |
| --- | --- | --- | --- |
| tproc_ctrl | 0 | Reset | Reset the tProc |
|  | 1 | Stop | Stop the tProc |
|  | 2 | Pause | Pause the tProc(Time continue Running) |
|  | 3 | Freeze | Freeze Time (tProc Runs, but time stops) |
|  | 4 | Run | Starts / Continue running the tProc |
|  | 5 | pStep | Debug Options |
|  | 6 | cStep |  |
|  | 7 | tStep |  |
|  | 10 | COND_set | Set External Condition Flag from |
|  | 11 | COND_clear | Clears External Condition Flag from |

Table 2: AXI-Registers

Register tproc_cfg is used to configure the behavioral of the tProc. It has 2 main function

- Configure the behavior of the LFSR.

- Configure the Memory Controller to Write/Read memories from Python.

| AXI-Register | Bit | Description |
| --- | --- | --- |
| tproc_cfg | 11 | Signal fifo_wave_full stops core (If 1, fifo_wave_full signal blocks the tProc) |
|  | 10 | Signal fifo_data_full stops core (If 1, fifo_data_full signal blocks the tProc) |
|  | 9..8 | LFSR Config |
|  | 4 | Memory Operation Data Source selection (0-AXIS, 1-REGISTERS (Single Read) ) |
|  | 3..2 | Memory Bank Selection for Operation (01-Pmem , 10-Dmem , 11-Wmem) |
|  | 1 | Memory Operation selection (0-Read, 1-Write) |
|  | 0 | Start Memory Operation (1-Start) To make a new, Go to 0 first. |

Table 3: tproc_cfg Register Description

In order to read or write one of the memories using the DMA, first the configuration should be done, and after that, the DMA transfer can be done. Values for the AXI-Register tproc_cfg for different operation are shown in the table below.

The start logic triggers on a rising edge:

A low level arms the trigger (transitions from "end" to "init" state).

A high level fires the trigger (starts the program).

For example, to Write 512 values to the Data Memory, starting in the address 127 , First the tProcessor should be configured and after that the DMA can be started as follow:

- Write a 512 in AXI-register mem_len

- Write a 10  in AXI- register mem_addr

- Write a 11 (1011b)  in AXI- register tproc_cfg

  - tproc_cfg[0] (START) should be 1, to start the machine

  - tproc_cfg [1] (Memory Operation)should be 1, to set a Write operation

  - tproc_cfg [3:2] (Memory Bank Selection)should be 10, to select Data Memory

- Make DMA Transfer

- Write a 0  in AXI- register tproc_cfg

PYTHON EXAMPLES:

print('Program FPGA')

soc = PfbSoc('./tproc.bit', ignore_version=True, init_clks=True)

print('Write Program Memory')

wr_buff = program_memory

soc.tproc.tproc_cfg       = 7

dma.sendchannel.transfer(wr_buff)

dma.sendchannel.wait()

soc.tproc.tproc_cfg       = 0

print('Reset tProc')

soc.tproc.tproc_ctrl      = 1 # RST

print('Start Processing')

soc.tproc.tproc_ctrl      = 16 # PLAY

print('Set External Condition')

soc.tproc.tproc_ctrl      = 1024 # COND set

print('Stop Processing')

soc.tproc.tproc_ctrl      = 2 # STOP

print('Read Data from tProc')

tproc_time_usr = soc.tproc.time_usr

print('TIME_USR: ', time_usr)

tproc_rand = soc.tproc.rand

print('RAND: ', tproc_rand)

tproc_d1 = soc.tproc.tproc_ext_dt1_o

print('TPROC_EXT_DT_O: ', tproc_d1)

tproc_d2 = soc.tproc.tproc_ext_dt2_o

print('TPROC_EXT_OP_O: ', tproc_d2)

print('Continue Processing')

soc.tproc.tproc_ctrl      = 16 # PLAY

print(Read Data Memory')

soc.tproc.mem_addr        = 0

soc.tproc.mem_len         = 522

soc.tproc.tproc_cfg       = 9

dma_acc_recv.transfer(rd_buff)

dma_acc_recv.wait()

soc.tproc.tproc_cfg       = 0

print(rd_buff)

## Core

The t-core, with a 5-stages pipeline Von Neumann architecture, is especially designed for high performance timed applications.

### 7Register Organization

The tProc-V2 has 3 register Banks.  For data, wave and special function registers. Also has Register and a stack for Program Counter, and Flags and Condition registers.The processor contains:

- 32 General purpouse 32-bit registers (dreg).

- 16 Special Function Registers (sreg)

- Waveform Parameter Register: The 168 bit register is r_wave each of the 32 or 16 bits registers are wreg

- Program Counter (PC)

- Time Condition Flag Register (TCF)

- External Condition Flag Register (ECF)

- Flags Register

Table 5: tProcesor-V2 Register Organization

### Special Function Registers

- ZERO 		> Zero value register. In assembler code s0 or zero

- RAND 		> 32-bit Pseudorandom number. In assembler code s1 or rand

- ARITH_LOW 	> Lower 32-bit of Arithmetic Operation. In assembler code s2 or arith_l

- ARITH_HIGH 	> Higher 32-bit of Arithmetic Operation. In assembler code s3 or arith_h

- DIV_QUOTIENT 	> 32-bit quotient of Division Unit integer division. In assembler code s4 or div_q

- DIV_REMAINDER 	> 32-bit remainder of Division Unit integer division. In assembler code s5 or div_r

- STATUS 		> Status register. In assembler code s6 or status

  - bit_0 > Division Unit Ready

  - bit_1 > New data in Data Port in

  - bit_2 > Data FIFO FULL

  - bit_3 > Wave FIFO FULL

- TPROC_EXT_DT1_I 	> Data 1 from Python to Processor. In assembler code s7 or ext_i1

- TPROC_EXT_DT2_I	 > Data 2 from Python to Processor. In assembler code s8 or ext_i2

- PORT_LSW 		> Lower 32-bit of Port Read Operation. In assembler code s9 or port_l

- PORT_MSW	> Higher 32-bit of Port Read Operation. In assembler code s10 or port_h

- TIME_USR 		> User Time value, Higher 32-bit of Port Read Operation. In assembler code s11 or tuser

- TPROC_EXT_DT1_O	> Data 1 from Python to Processor. In assembler code s12 or ext_o1

- TPROC_EXT_DT2_O	> Data 2 from Python to Processor. In assembler code s13 or ext_o2

- OUT_TIME 		> Time register for port writing. In assembler code s14 or r_time

- PC_JMP_ADDR	> Address register for PC branch. In assembler code s15 or r_addr

### Random Number Generation

Special Function Register rand (s1) is the output of a LFSR. The LFSR is a Fibonacci Serie. The Polynomial implemented is x31+x21+x1+x0 which have a maximum-length. A class in python was implemented that calculates the series for simulation. The design was selected based on the Xilinx Application Note XAPP052. Figure 5: LFSR schematic

Figure 5: LFSR schematic

The LFSR has 4 working modes, and is configured with the AXI-Register CFG = tproc_cfg[9..8]. Table 6: LFSR Configuration Values, shows the configuration values for the working modes.

| CFG | State | Description |
| --- | --- | --- |
| 00 | STOP | Keep the current Number |
| 01 | Free Running | Each clock, LFSR Change |
| 10 | Change When read | When s1 is read LFSR made a step |
| 11 | Change Manually | When s0 is written LFSR made a step |

Table 6: LFSR Configuration Values

## Peripherals

### Time Comparator

This block compares the current time with a preloaded value. It takes 2 clock cycles to get the result, so the TCF will be set 2 clock cycles after the desired time has arrived. To command TIME set_cmp <Value> is used load the comparison value.

### Arithmetic Unit

This block performs the ( D ± A) * B ± C   Arithmetic operation in 2 clock cycles. The 64-bit result value is stored in Special Function Registers arith_l(s2) and arith_h(s3).

### Division Unit

This block calculates the integer quotient and the remainder of 2 32-bits integer. The Quotient is a 32-bit value div_q(s4) and the remainder is a 32-bit register div_r (s5). The source data for the Numerator is always a register and the denominator can be a register or a Literal Value. This unit takes 32 clock cycles to perform the division. At the beginning of the division the external Condition Flag (ECF) is cleared and at the end the flag is set.

## Timing & Dispatcher

The time is measured in clock cycles and is implemented with an always running 48-bit counter. The time of each tick depends on the clock period, that depends on the configuration of the DAC. With a clock of 256Mhz, the period is 3.9065ns. If the counter increment is 32 counts, it means 125ns. The maximum time until the counter overflows with a Clock of 500Mhz (This clock depends on the DAC Configuration) is:

248*2ns≅562.949 Seconds ≅ 9.382 Minutes ≅ 156 Hours

The abs_time is a 48-bit register, and the comparison is done with a 48-bit comparator. So, in order to get the 48-bit for the time, from the 32-bit of the processor, a 48-bit ref_time  register is used to set the starting point of the experiment (t0) and to get the 48-bit out_time for the output timing signals. The user can set the reference value to move the t0 of the experiment a certain time.

out_time = ref_time + user_time

abs_time	: The absolute time is a 48-bit counter that run at the DAC frequency.

user_time	: The user time is the time that the user set on the instructions. Is a 32-bit value

ref_time	: The reference time is the base value (or offset ) used to move the user time to a certain “zero”.

out_time	: The out time is the 48-bit real time for the port to be written.  This value is used to be compared with the abs_time.

The abs_time starts in 0 and can be reset with an Assembler Instruction (TIME rst) or from Python (Writing in AXI-Register ctrl_reg). The dispatcher is continuously reading the WaveFIFO and DataFIFO and comparing the Output Time of the signals with the current abs_time . When times are equals (abs_time = out_time), or the time to write the signal has already passed (abs_time > out_time) the Wave or the DATA are written to the corresponding port.

The fastest dispatcher throughput is one data every 3 clock cycles. The dispatcher verifies for time equal and for time greater.

With a clock of 2.5ns the maximum time for signal update is 7,5ns.

# Python Interface

The three memories can be written and read with the DMA interface. The AXI interface have 4 registers to read data from the DMEM

Write (and read) to (from) memories from the PS interface can be done in two different ways. Using a 256-bit DMA controller or with an AXI command.

Using the DMA>

When DMA is used, the start memory address and the data length must be configured on the registers MEM_ADDR and MEM_LEN. After that, the operation can be started writing the appropriate value on the TPROC_CFG register.

When DMA is used, the start memory address and the data length must be configured on the registers MEM_ADDR and MEM_LEN. After that, the operation can be started writing the appropriate value on the TPROC_CFG register.

Read DMEM with AXI command>

The TPROC_CFG is used to config the source / destination of the read/write command, as shown in Table 4:tproc_cfg values for Memory  Write and Read operations.

When single value is read, the memory address must be selected on the register MEM_ADDR. After that, the operation can be started writing the appropriate value on the TPROC_CFG register. And the read memory value will be stored on MEM_DT_O register. When single value is written, the memory address must be selected on the register MEM_ADDR and the data written in MEM_DT_I. After that, the operation can be started writing the appropriate value on the TPROC_CFG register.

Table 4:tproc_cfg values for Memory  Write and Read operations

COmands.

Reset

soc.tproc.reset()

Stop

pauser

# Instruction Set

The tProc-V2 has 15 instructions and the capability to perform multiple tasks in the same instruction.

## Summary

| Operation | Assembler |
| --- | --- |
| No Operation | NOP |
| Test Register Value with aa ALU operation and update Flags. | TEST <op> |
| Write Register (Rd: Data Register, Special Register or Wave Parameter Register) from Source (Sources : ALU Operation, Data Memory, WaveParam Memory or Immediate value) | REG_WR <Rd> <Source> |
| Update the value of the Special Function Registers 8 and 9 to the value of the selected port. | DPORT_RD <Port> |
| Write data to Data Memory. | DMEM_WR [Address] <Source> |
| Write Wave Memory. Copy the r_wave register value to the specified address in the Wave Memory. | WMEM_WR [Address} |
| Write a 32-Bit Data to specific Data Port at Specific time (Time can be an Immediate Value or a registered Value) | DPORT_WR <Port> <Source> <Time> |
| Write a Wave to specific Wave Port at Specific time (Time can be an Immediate Value or a registered Value) | WPORT_WR <Port> <Source> <Time> |
| Conditional JUMP to a specific Address (Address can be a Label, an Immediate Value or a Register) | JUMP [Address] |
| Function CALL to a specific Address. CALL can be Nested Up to 4 times | CALL [Address] |
| RETURN from specific CALL to previous PC. | RET |
| Command to control time_ref and time_abs in Processor. Reset Absolute Time (time_abs = 0), Change Time Ref (time_ref = Imm_value), Increase Time Ref (time_ref = time_ref + Imm_value) | TIME <Option> <Value> |
| Make ( D ± A) * B ± C Arithmetic operation in 2 clock cycles. 64-bit result value is stored in Special Function Registers 2 and 3 | ARITH <Option> <Sources> |
| Calculate Quotient and the Reminder of the division. It takes 32 clock cycles. Is done by a co-processor | DIV <Num> <Den> |
| Modifies the Value of the Condition Flag. | COND <Set/Clear> |

## Instructions options

All instructions have optional arguments to configure and perform different tasks. The user can specify to execute a second instruction in the same Scan execute a second optional instruction in the same command. There are Second Data Wave and Port Write instructions also a TEST can be done.

| Description | Option |
| --- | --- |
| ALU Operation | -op(<Operation>) |
| Update Flags | -uf |
| Conditional Command Execution | -if(<Condition>) |
| Specify the Time with an Immediate Value | @<Time> |
| Add a Write Register Task to the command | -wr(<Dest> <Source>) |
| Add a Write Port Task to the command | -wp(<Port>) |
| Add a Write WaveMemory Task to the command | -ww |

Table 7: Assembler Instruction Options

ASSEMBLER EXAMPLES:

| REG_WR r0 op -op(r1-r2) | > Stores in r0 = r1-r2 |
| --- | --- |
| REG_WR r0 op -op(r1-r2) -uf | > Stores in r0 = r1-r2 and update S and Z Flags |
| JUMP [LABEL] -if(NZ) | > Jump to LABEL Address if Flag is NonZero |
| WPORT_WR r_wave p1 @100 | > Write r_wave to Wave Port 1 at Time 100 |

## ALU Operation  < -op >

The Arithmetic & Logic unit can perform 15 different operations. When executing instruction REG_WR all 15 operations are available, but when executing ALU operations as a Second option, only 4 are available. Table 8: ALU Operations shows all operations and in gray the 4  for SDI.

| Code |  |  |  | Operation |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | ADD | Addition |
| 0 | 0 | 0 | 1 | SUB | Subtraction |
| 0 | 0 | 1 | 0 | AND | Logical bitwise AND |
| 0 | 0 | 1 | 1 | ASR | Arithmetic Shift Right |
| 0 | 1 | 0 | 0 | ABS | Absolute Value |
| 0 | 1 | 0 | 1 | MSH | Most Significative Half (16MS Bits) |
| 0 | 1 | 1 | 0 | LSH | Least Significative Half (16LS Bits) |
| 0 | 1 | 1 | 1 | SWP | Swap Half Word (16LSB-16MSB) |
| 1 | 0 | 0 | 0 | NOT | Logical bitwise NOT |
| 1 | 0 | 0 | 1 | OR | Logical bitwise OR |
| 1 | 0 | 1 | 0 | XOR | Logical bitwise XOR |
| 1 | 0 | 1 | 1 | CAT | Concatenate 2 LSB 16Bits |
| 1 | 1 | 0 | 0 | RFU | Reserved Future Use |
| 1 | 1 | 0 | 1 | PAR | Parity |
| 1 | 1 | 1 | 0 | SL | Logic Shift Left |
| 1 | 1 | 1 | 1 | SR | Logic Shift Right |

Table 8: ALU Operations

| Operation | Description | Assembler Code |
| --- | --- | --- |
| NONE | Copy the value of a Register | -op(r0) |
| ADD | Register plus Immediate value | -op(r1 + #7) |
|  | Register Plus Register value | -op(s1 + r2) |
| SUB | Register minus Immediate value | -op(r1 - #7) |
|  | Register minus Register value | -op(s1 - r2) |
| ASR | Arithmetic Shift Right Immediate Amount (MSB completed with Sign bit, up to 15 shifts ) | -op(r1 ASR #7) |
|  | Arithmetic Shift Right Immediate Amount | -op(s1 ASR r2) *(r2 < 16) |
| ABS | Absolute Value | -op(ABS r1) |
| MSH | Most Significan HalfWord | -op(MSH r2) |
| LSH | Least Significan HalfWord | -op(LSH r3) |
| SWP | Swap HalfWord | -op(SPW r4) |
| CAT | Concatenate 2 HalfWord 16 Bits | -op(r5 CAT r6) |
| PAR | Parity Check | -op(PAR r7) |
| SL | Shift Left (from 0 to 15 shifts) | -op(r8 SL r3) |
| SR | Shift Right (from 0 to 15 shifts) | -op(r9 SR r3) |
| NOT | NOT Register | -op(NOT r1) |
| AND | Register AND Immediate | -op(r1 AND #7) |
|  | Register AND Register | -op(s1 AND r2) |
| OR | Register OR Immediate | -op(r2 OR #7) |
|  | Register OR Register | -op(r2 OR r2) |
| XOR | Register XOR Immediate | -op(r3 XOR #7) |
|  | Register XOR Register | -op(r3 XOR r2) |

Table 9: ALU Operations Examples

## Conditional Execution < -if() >

All Data movement instructions contains a condition field which determines whether the CPU will execute them. This removes the need for many branches, which stall the pipeline (2 cycles to refill) This Allows very dense in-line code, without branches (but more memory is used). User must decide if  the time penalty of not executing several condition instructions has more or less overhead of the branch.

The Condition Field is a 3 Bits field, what gives up to 7 conditions. There are 4 ALU related Condition, two Time related and one External Condition. To use the condition in the Assembler just add the -if() option

By default, data processing operations do not affect condition flags. To cause the condition flag to be updated, the -uf (update Flag) option should be included in the instruction. Table 10: Conditions shows a list of the 15 conditions.

Table 10: Conditions

ASSEMBLER EXAMPLE:

| REG_WR r0 imm #0 | > r0 = 0 |
| --- | --- |
| REG_WR r0 op -op(r1+r2) -if(Z) | > r0 = r1+r2 IF Z flag is set |
| JUMP [LABEL] -if(Z) | > Jump to LABEL Address if Flag is NonZero |

The Time Condition is generated by a comparator that compares the user_time with the desired comparison time. To set the condition time, use the command TIME set_cond. If  the user_time is greater that the condition time the Condition is True, otherwise is False. This is used to make the Tproc wait for a specific time.

The External Condition can be set and clear by the Tproc (with the COND command), By Python (Writing in the TPROC_CTRL Reg bit 10 SET bit 11 Clear),  is used to control the status of the divider and also is set to one if a New Data is captured in some of the Input PORTS.

The External Condition can be set by>	tProc / Python / Port / Divider

The External Condition can be clear by> tProc  / Python / Divider

ASSEMBLER EXAMPLE:

| REG_WR r0 op -op(r2-#10) | > r0 = r2-10 no Flag Update |
| --- | --- |
| REG_WR r0 op -op(r2-#10) -uf | > r0 = r2-10 and SET FLAGS |
| COND set | > Set External Condition Flag |
| COND clear | Clear External Condition Flag |
| DIV r10 r11 | > Clear External Condition Flag makes the Division (r0 / r11) and set the FLAG when finish. |
| TIME set_cmp r4 | > Set the Time Condition value to the value stored in register 4. |

## Dual task Instructions. < -wr(), -wp(), -ww >

Depending on the options used, the instruction can also execute a second optional task in the same command. There are Second Data (-wr() ) Wave ( -ww ) and Port ( -wp() ) tasks also a TEST (-op() -uf ) can be done.

| Command | Second Data Task | Second Wave Task | Second Port Task | Conditional Execution | Update Flag |
| --- | --- | --- | --- | --- | --- |
| NOP |  |  |  |  |  |
| TEST |  |  |  |  | YES |
| REG_WR |  |  |  | YES | YES |
| DPORT_RD |  |  |  |  |  |
| DMEM_WR | YES |  |  | YES | YES |
| DPORT_WR | YES | YES |  | YES | YES |
| REG_WR | *YES | YES | YES |  | YES |
| WMEM_WR | YES | YES | YES |  | YES |
| WPORT_WR | YES | YES |  |  |  |
| JUMP | YES |  | YES | YES | YES |
| CALL | YES |  |  |  | YES |
| RET | YES |  |  |  | YES |
| TIME | YES |  | YES |  | YES |
| ARITH |  |  |  |  |  |
| DIV |  |  |  |  |  |
| COND |  |  |  |  |  |

*When REG_WR source is dmem, a TEST can be done to update the flags.

Second Data Instruction can be:

- Register Write: To enable second register write, the option -wr(<Rd> <Source>) should be added to the command.

- Register Test: To enable a register test, the operation -op() and the -uf option should be added.

### Write Register

The write register option is added with the -wr(dest, source) it has 2 operands destination register and source. Destination register can be a (dreg, sreg or wreg) register. Source can be imm or op. Only one immediate (Literal) value can be used per instruction and up to 2 register values. As shown in Table 12: Data Source Format, if no register value is used, Immediate value is a 32-bit value. If a register is used, Immediate value is a 24-bit value if 2 registers are used the immediate value is a 16 bit value.

ASSEMBLER EXAMPLES:

| -wr(r0 imm) #5 |  | > SDI: Write the value 5 in r0 |
| --- | --- | --- |
| JUMP [LABEL] -wr(r1 op) -op(rand) | > SDI: Copy the random value in r1 |  |
| DMEM_WR [r0+r1] imm #5 -wr(r4 op) -op(r2+r3) | > Write to Data Memory Address [r0-r1] the literal value 5 AND Write r4 the operation r2+r3 |  |
| REG_WR |  |  |

### Write port

ASSEMBLER EXAMPLES:

| DPORT_WR p0 op -op(r0+#1) -wr(r0 op) | > Write the operation r0+1 to Data Port 0 AND to r0 |
| --- | --- |
| DMEM_WR [&3] op -op(r1-r2) -wr(r3 imm) #1 | > Write to Data Memory Address 3 the operation r1-r2 AND Write r3 the Immediate Value 1 |
| REG_WR r_wave wmem [&3] -wr(r5 op) -op(r6-#1) -uf -wp(r_wave) -p8 | > Write to r_wave the wave stored in Wave Memory address 3 AND Stores in r5 = r6-1 AND Write r_wave (the new updated value from memory) to Wave Port 8 at time specified in r_time |
| JUMP [STAT_ADDR] -if(S) -wr(r1 op) -op(r1-#1) -uf | >Jump to START_ADDR if Sign flag (Negative) AND Write r1 = r1-1 (Decrement r1)AND Update S and Z Flags |

# Assembler

The tproc-v2 python assembler translate sources files in the assembly language with the format specified in this document to a bit file ready to be loaded to the Program File. This translation process is called assembly.

## Statements

This section outlines the types of statements that apply to assembly language.

Each statement must be one of the following types:

- An empty statement is one that contains nothing other than spaces, tabs, or formfeed characters. Empty statements have no meaning to the assembler. They can be inserted freely to improve the appearance of a source file or of a listing generated from it.

- A label consists of a symbol starting by a colon (:). When the assembler encounters a label, it assigns the value of the location counter to the label.

- A comment can be inserted to improve the code.  Appending a comment at the beginning or the end of the statement by preceding the comment with a double slash (//).

- A machine command statement is a mnemonic representation of an executable machine language instruction to which it is translated by the assembler. It consists of an instruction, optionally followed by operands.

- A directive statement is an instruction to the assembler that not necessarily generates code.

The Statements are given by the following expressions

- EMPTY    = r'^$'

- LABEL    = r'[A-Za-z0-9_]*(?=\:)'

- COMENT   = r'^\s*//'

- COMMAND  = r'^[A-Z_]+'

- DIRECTIVE = r'(?<=\.)[A-Z]*'

## Directives

#### .ALIAS

Alias names for the registers can be created using the .ALIAS directive

ASSEMBLER EXAMPLES:

| .ALIAS addr_aux r0 REG_WR addr_aux #100 | > Set the name for Register r0 to addr_aux > Stores in addr_aux (r0) value 100 |
| --- | --- |
| .ALIAS step_time r1 REG_WR step_time #256 REG_WR r_time op -op(r_time +step_time) | > Set the name for Register r1 to step_time > Stores in step_time (r1) value 256 > Increments r_time in step_time (256) |

#### .END

Program can be finished with the .END directive, this will avoid the program counter to go further. This directive add a JUMP [HERE] instruction to the program.

## Machine instruction Syntax

This section describes the instructions that the assembler accepts. The detailed specification of how the instructions operates is not included.

The following list describes the two main aspects of the tproc-v2 assembler:

- The 3 banks of registers use a prefix to distinguish them from symbol names. (r, s, or w).

  - General purpose registers start with r

  - Special Function registers start with s

  - Wave Parameters registers start with w

  - Special Function and wave Register have alias names.

- Instructions with two operands use the left one as the destination and the right one as the source.

  - Instructions Options can be written in any order

### Operands

Two kinds of operands are generally available to the instructions: register and immediate. The assembler always assumes it is generating code for a 32-bit signed integer values. For some dual operations, the immediate value can be a 24-bit or a 16-bit integer.

### Addressing modes

- The Processor has 4 addressing modes for the Data Memory address

  - Literal > The address is an Immediate Value

  - Register > The address is stored in a Register

  - Indexed Literal > The address is the Sum of a Register Value and a Immediate Value

  - Indexed Register > The address is the Sum of 2 registers.

- The Processor has 2 addressing modes for the WaveParam Memory address

  - Literal > The address is an Immediate Value

  - Register > The address is stored in a Register

- The Processor has 2 addressing modes for Branch instructions

  - Literal > The address is an Immediate Value

  - Register > The address is the register s15 r_addr

### Instruction description

This section describes the tproc-v2 instruction syntax. The assembler generate code for a 32-bit integer values for data and 16-bit values for address.

- All Instructions are UPPERCASE.

- Comments starts with //.  In Line comments are allowed.

  - Avoid using “:” in the comment, it will be interpreted as a LABEL

- Labels end with “:” Contains numbers, UPPERCASE, lower case and “_” symbol. No space is allowed in Labels.

- Literal Values start with “#” symbol. Binary Values start with “#b”

- General Purpose registers start with r

- Special Function Registers start with s

- Wave Parameter registers start with w

- Conditions are UPPERCASE

- Address should go between square brackets “[]”

- Literal Address start with the & symbol.

- Address can only be General Purpose registers

- Ports start with p

- Time starts with @

- Instruction options, second instructions and register alias are lowercase.

| CORRECT | INCORRECT |
| --- | --- |
| REG_WR r0 imm #0 | reg_wr r0 imm #0 |
| // Comment | /* Comment (Only one /) |
| REG_WR r0 imm #0 // Set r0 to Zero | REG_WR r0 imm #0 --Set r0 to Zero (Not //) |
| LOOP_START: | :LOOP_START (Not ends with :) |
| OPTION_2_reset: | :OPTION_#2 (# not allowed) |
| TIME set_cmp #100 | TIME set_cmp 100 (# Missing) |
| REG_WR r0 op -op(w0 AND #b101) | REG_WR r0 op -op(wo AND #b5) (with #b only 0 and 1) |
| REG_WR r0 imm #0 | REG_WR reg0 imm #0 (is r0) |
| REG_WR r0 op -op(w0+s1) | REG_WR r0 op -op(W0+S1) (w and s lowercase) |
| REG_WR r_freq op -op(w0+#1) | REG_WR w_freq op -op(W0+#1) (Alias is r_freq, reg is w0) |
| JUMP [LABEL] -if(NZ) | JUMP [LABEL] -if(nz) (is NZ) |
| DMEM_WR [r1] imm #b0110100 | DMEM_WR &1 imm #b0110100 (Not []) |
| WMEM_WR [&0] | WMEM_WR [#0] (Not &) |
| DMEM_WR [r2+&2] op -op(r_freq) | DMEM_WR [s2+&2] op -op(r_freq) (Address only dreg) |
| DPORT_RD p0 | DPORT_RD port0 (is p0) |
| DPORT_WR p1 op -op(r0+#1) | DPORT_WR &1 op -op(r0+1) (is p1Missing #1) |
| WPORT_WR wmem [&2] p2 | WPORT_WR wmem [s2] p2 (Address only dreg) |
| WPORT_WR r_wave p3 @99 | WPORT_WR r_wave @99 (Missing Port) |
| WPORT_WR wmem [&2] p2 @125 | WPORT_WR wmem p2 @125 (Missing wmem address) |
| REG_WR r_wave wmem [&3] -wr(r5 op) -op(r6-#1) | REG_WR R_wave wmem [&3] -WR(r5 OP) -OP(r6-#1) |

## Instruction Description

Instructions are 72-bit wide. The 72 Bits are divided in 16-bits OP_CODE and 56-bits OP_DATA.

Figure 6: Instruction OPCODES

Header : These 3 bits indicate the type of instruction. There are 8 Types of instructions as shown in Table 11: Header Instruction Type.

Table 11: Header Instruction Type

AI [Address Immediate]: This bit indicates if the Address Source [0] (Bits OPData [55:45]) is an immediate value or a register stored value. 0-Register 1-Immediate

Table 12: Data Source Format

DF [Data Format]: These two bits are used to define the Data Source format. A Zero indicates two registers will be used. A One indicates two registers and an Immediate 16Bits value will be used. A Two indicates one register and an Immedite 24Bits will be used. A Three indicates an Immediate 32Bits value is used. As shown in Table 12: Data Source Format

COND[Condition]: Some Data instruction and Branch instructions contains a condition field wich determines whetheIndicate the Condition for Instruction execution. Some Instructions are executed only of the Condition is md

reg_src [Register Data Source]: Indicate the source of the data to the register to be written.

uf [Update Flag] :

wr [Register Data Immediate] : wr Indicate if theres is a optional write register operation.

rdi [Write Register] : RDI indicates if the data source to the register is immediate or Alu 0-Register 1-Immediate.

alu_op [Arithmetic Logic Unit Operation] : Indicates the operation to be done by the ALU. If is a write_reg operation there are 16 operations, otherwhise (for a second option write reg) 4 operations are available.

### Configuration Instructions.

Instruction with HEADER = 000. This set of instructions does not change the value of any data (Register, Memory, or Port). Just the Flags.

#### NOP

This instruction executes NO OPERATION. It takes One clock cycle

ASSEMBLER EXAMPLE:

| NOP | > No Operation |
| --- | --- |

#### TEST

This instruction performs an ALU operation and update the FLAG values, but to result is written. The operation can be done between 2 registers or a register and an Immediate Value. Possible ALU operations are +, -, AND and ASR.

ASSEMBLER EXAMPLES:

| TEST -op(r3 - #3) | > Update the flags with the Instruction r3-3 |
| --- | --- |
| TEST -op (r4 AND #b11) | > Update the flags with the Instruction r3 AND 3 |

### Register Instructions

Instruction with HEADER = 100. This set of instructions change Register values (General Purpose, Special Function and WaveParam)

#### REG_WR

This instruction performs a register write, the destination register can be a sreg, a dreg, wreg or r_wave. Data Source can be an ALU operation (00), a 32-bit Data Memory (01), a 128-bit WaveParam (10) or an Immediate Value(11).

If Source is ALU operation (00), no SDI (Second Data Instruction) can be done. Depending on DF Field, the Operation can be done between 2 registers (dreg, sreg or wreg) or a register(dreg, sreg or wreg) and a 24-bit Immediate Value. This instruction is conditional

If Source is Data Memory (01) or Immediate Value(11), a TEST operation can be done as a SDI. This instruction is conditional

If Source is WaveParam Memory(10), the destination register is the r_wave. A Write Data Register operation can be done as a (Second Data Instruction) SDI.  Write Wave, and Write Port can be done as a Second Wave (SWI) and Port (SPI) Instruction. The SPI Source can be the r_wave or the WaveParam Memory Data. This instruction is NOT conditional

Sources

- op > ALU Source

- imm > Immediate Value  Source

- dmem > Data Memory Source (Read Data memory Command)

- wmem > WaveParam Memory Source (Read WaveParam memory Command)

- label > Used to store in a register (usually r_addr) the address of a label in the program memory.

ASSEMBLER EXAMPLES:

| REG_WR r0 imm #0 | > Stores in r0 = 0 |
| --- | --- |
| REG_WR r1 op -op(rand) | > Stores in r1 random number |
| REG_WR r2 op -op(r1 ASR #1) | > Stores in r2 = r1 Arithmetic shift right once |
| REG_WR r3 dmem [&10] | > Stores in r3 = Data Memory Address 10 |
| REG_WR r4 dmem [r2 + &10] | > Stores in r5 = Data Memory Address r2+10 |
| REG_WR r_wave wmem [&0] | > Stores r_wave = WaveParam Memory Address 0 |
| REG_WR r_addr label PROC_1 | > Stores in r_addr the address of the label PROC_1. |

### Memory Instructions

Instruction with HEADER = 101. This set of instructions write Memory  values (Data and WaveParam)

#### DMEM_WR

This instruction performs a Data Memory Write instruction, the destination address is rsA[0]+rsA[1], where rsA[0] can be a register address or a literal value, depending on the AI bit. Register rsA[1] can only be a dreg.

A Write Data Register operation can be done as a (Second Data Instruction) SDI.

This instruction is conditional.

The immediate address is stored in rsA[0], so the maximum immediate address is 11 bits.

ASSEMBLER EXAMPLES:

| DMEM_WR [&0] imm #10 | > Stores in Data Memory Address 0, Value 10 |
| --- | --- |
| DMEM_WR [r1] op -op(r0 + #1) | > Stores in Data Memory Address pointed by r1 value of the operation r0 + 1 |
| DMEM_WR [r1+r3] op -op(r_freq) | > Stores in Data Memory Address pointed by r1+r3, value of the freq parameter(w0) |
| DMEM_WR [r1+&4] imm #10 -wr(r5 op) -op(r2+r3) | > Stores in Data Memory Address pointed by r1+4, value 10 and writes in r5 = r2 + r3 |

#### WMEM_WR

This instruction performs a WaveParam Memory Write instruction, the destination address can be a Literal value or a register value, depending on the AI bit. The Source of the data is always the 128-bit r_wave register.

A Write Data Register or TEST operation can be done as a (Second Data Instruction) SDI.

A Write Port can be done as a Second Port Instruction(SPI). The SPI Source can be the r_wave or the previous WaveParam Memory Data on the address selected (Before the Write).

This instruction is NOT conditional

ASSEMBLER EXAMPLES:

| WMEM_WR [&0] | > Stores in WaveParam Memory Address 0, the r_wave value |
| --- | --- |
| WMEM_WR [r1] -wr(r_freq op) -op(r_freq+#10) | > Stores in WaveParam Memory Address pointed by r1, the r_wave value, and increment the freq value in 10. (Updates the r_wave) |
| WMEM_WR [&3] -wr(r_gain op) -op(r_gain+#25) -wp(r_wave) -p3 | > Stores in WaveParam Memory Address 3, the r_wave value, increment the gain value in 25 and writes the r_wave to port 3 |

### Port Instructions

Instruction with HEADER = 110. This set of instructions Operates with Ports (Data and WaveParam)

#### DPORT_WR

This instruction performs a Data Port Write. The destination port is always a literal value, and the time is always set with the special register r_time (s14). The Source Data can be a Literal or a ALU operation.

A Write Data Register or TEST operation can be done as a (Second Data Instruction) SDI.

This instruction is NOT conditional

ASSEMBLER EXAMPLES:

| DPORT_WR p0 imm #255 | > Writes the value 255 in port 0 at the time specified in r_time. |
| --- | --- |
| DPORT_WR p1 op -op(r1-#1) -wr(r1 op) -uf | > Writes the value of register r1 at the output port 1, decrements r1 and updates flags. |

#### DPORT_RD

This instruction performs a Data Port Read. The destination registers are always Special registers port_l(s8) and port_h (s9). The source port is a literal value.

A Write Data Register or TEST operation can be done as a (Second Data Instruction) SDI.

This instruction is NOT conditional

ASSEMBLER EXAMPLES:

| DPORT_RD p0 | > Reads the value of the Port 0. |
| --- | --- |
| DPORT_RD p1 -wr(r15 op) -op(r1+#1) -uf | > Reads the Port 1, stores in register r15=r1+1 and updates flags. |

#### WPORT_WR

This instruction performs a WaveForm Port Write. The destination port is always a literal value. The time can be the special register r_time (s14), or an Immediate Value, depending on the TI bit. The Source Data can be the 128-bit r_wave register or a WaveParam Memory value, addressed literal or registered.

If r_time is used as a time source, a Write Data Register or TEST operation can be done as a (Second Data Instruction) SDI.

This instruction is NOT conditional

ASSEMBLER EXAMPLES:

| WPORT_WR p0 r_wave | > Writes the r_wave in port 0 at the time specified in r_time. |
| --- | --- |
| WPORT_WR p1 r_wave @125 | > Writes the r_wave in port 0 at time 125 |
| WPORT_WR wmem [&2] p2 -wr(r1 op) -op(r1+#1) | > Writes the value of WaveParam Memory address 2 in port 2 at the time specified in r_time and increments r1. No update the flags. |

### Branch Instructions

Instruction with HEADER = 001. This set of instructions operates the Program Counter (PC).

#### JUMP

This instruction performs a change in the PC, making the program jump to a specified program address. The destination address can be a literal value or the special register r_addr (s1r), depending on the AI bit.

A Write Data Register or TEST operation can be done as a (Second Data Instruction) SDI.

This instruction is conditional

The assembler for this instruction has 4 reserved words.

- “HERE” is the same address than the JUMP instruction. Is used to stay on the same address (To create a Wait Intruction)

- “PREV” is the previous instruction. Jump to the address PC-1

- “SKIP” skip the next instruction. Jump to the address PC+2

- “r_addr” is used to jump to the address specified by the special function register r_addr(s15)

ASSEMBLER EXAMPLES:

| JUMP [%0] | > Unconditional jump to Address 0. |
| --- | --- |
| JUMP [HERE] -if(NEC) | > Stay in the same address until a external condition flag is set. |
| JUMP [LOOP] -if(NZ) -wr(r1 op) -op(r1-#1) -uf | > Jump to LOOP label if flag is NZ. Decrement r1 and updates the Flags. |

#### CALL

This instruction performs procedure CALL. Stores the PC in the Stack and jumps to the specified address. The destination address can be a literal value or the special register r_addr (s1r), depending on the AI bit.

A Write Data Register or TEST operation can be done as a SDI.

This instruction is conditional

#### RET

This instruction RETURNS from a procedure CALL.

A Write Data Register or TEST operation can be done as a SDI.

This instruction is NOT conditional

### Block Control Instruction

Instruction with HEADER = 111. This set of instructions allow the user to control the Peripherals Blocks.

#### TIME

This instruction perform time related instructions. It can modify the value of ref_time and abs_time. It has 4 diferent options.

- Reset the absolute time  abs_time = 0

- Set Reference time to a specific value ( ref_time =  Source Data)

- Increase Reference Time a certain value (ref_time =  ref_time +  Source Data)

- Set the Time comparator for Time Flag.

ASSEMBLER EXAMPLES:

| TIME rst | > Reset absolute time. |
| --- | --- |
| TIME set_ref r2 | > Set the reference time to value in r2 |
| TIME inc_ref #15750 | > Increment the reference time 15750(ref_time = ref_time + 15750) |
| TIME set_cmp #16800 | > Set the Time comparator to set the flag after the time 16800 |

#### COND

This instruction clears or set the external condition flag.

The flag is set with this command, with an arithmetc operation ends and when NEW data is written in some PORT. If the flag was set by a PORT. The only way to clear is by reading the data from the port with the DPORT_RD instruction.

ASSEMBLER EXAMPLES:

| COND set | > set the External condition flag. |
| --- | --- |
| COND clear | > Clears the External condition flag. |

#### ARITH

This instruction perform an arithmetic operation with the FPGA DSP. Make ( D ± A) * B ± C   Arithmetic operation in 2 clock cycles. 64-bit result value is stored in Special Function Registers s2 and s3.

With values a, b, c, d, e of the Operation Field in the OP_Code, user selects the operation. A List of operation codes are shown in Table 13: ARITH Operation Codes.

Table 13: ARITH Operation Codes

| OPERATION |  | Assembler Command | Operation Executed |
| --- | --- | --- | --- |
| T | A * B | ARITH T r1 r2 | r1 * r2 |
| TP | A * B + C | ARITH TP r1 r_freq rand | r1 *r_freq+rand |
| TM | A * B - C | ARITH TM r1 s2 r3 | r1 * s2 – r3 |
| PT | (D+A) * B | ARITH PT r2 r1 r_gain | (r2 + r1) * r_gain |
| MT | (D-A) * B | ARITH PT r1 r2 r3 | (r1 – r2) * r3 |
| PTP | (D+A) * B + C | ARITH PTP r1 rand r3 r4 | (r1 + rand) * r3 + r4 |
| PTM | (D+A) * B - C | ARITH PTM r1 r2 r3 r4 | (r1 + r2) * r3 - r4 |
| MTP | (D-A) * B + C | ARITH MTP r1 s2 w3 r4 | (r1 - s2) * w3 + r4 |
| MTM | (D-A)*B-C | ARITH MTM r1 r2 r3 r4 | (r1 - r2) * r3 - r4 |

ASSEMBLER EXAMPLES:

| ARITH T r1 r2 | > Makes r1 Times r2. |
| --- | --- |
| ARITH PT r1 s2 w3 | > Makes (r1 Plus s2) Times w2 (r1+s2)*w2 |
| ARITH MTP r1 s2 w3 r4 | > Makes (r1 Minus s2) Times w3 Plus r4 (r1-s2)*w3+r4 |

#### DIV

This instruction 	configures the Division Unit Co-Processor to make a division. The destination registers are always Special registers div_q(s4) for the quotient and div_r (s5) for the remainder of the division.

The source data for the Numerator is always a register and the denominator can be a register or a Literal Value.

ASSEMBLER EXAMPLES:

| DIV r1 r2 | > Makes r1 / r2. |
| --- | --- |
| DIV r1 #100 | > Makes r1 / 100 |
| DIV #100 r1 | > Makes r1 / 100 (Immediate is always the denominator) |

## Assembler Examples

### WAIT

To wait until specific time

| TIME set_cmp #100 JUMP [HERE] -if(NTC) | > WAIT TO 100, using the External comparator |
| --- | --- |
| TEST -op(tuser - #150) JUMP [PREV] -if(S) | > WAIT TO 150, using the internal ALU |
| TIME set_cmp #200 JUMP [SKIP] -if(TC) JUMP [PREV] | > WAIT TO 200, using the External comparator |

# Architecture details

##### Configurable Parameters

- IN_PORT_QTY > Data Port In quantity (Up to 8)

- OUT_DPORT_QTY > Data Port Out quantity (Up to 4)

- OUT_WPORT_QTY > Wave Port Out quantity (Up to 16)

- PMEM_AW: Program Memory Address width. This parameter defines the Program Memory Size.

- DMEM_AW: Data Memory Address width. This parameter defines the Data Memory Size.

- WMEM_AW: Wave Memory Address width. This parameter defines the Wave Memory Size.

- REG_AW > Data Registers (Up to 32)

## Pipeline Stages

Fetch – Decode – ReadReg - Execute_First - Execute_Second-Write_Back

Fecth > Get the Instruction From Memory

Decode > Generates all the Control signals from the OP_CODE and Access the Register Bank and Get all the Data signals

Read > Get the Data from the Pipeline if was previously processed.

Execute_First > Calculate the ALU Operation, Calculates the DataMem and Wmem Address. Update the ALU FLAGS

Execute_Second > Read/Write Data Memory or Wave Memory

Write_Reg > Write the Final Result to the register Bank or to the Output Port.

## Signals Names and Convention

## Debugging

TIMING DETAILS>

Time counter start 2 cycles after the Processor starts.

Timing Control Instructions must cross clock domain, so it takes 2 clock to go, and 2 to return to execute. So. DO NOT execute 2 TIME instruction with less than 5 clocks on the slowest Domain.

Hazards NOT Verified FOR DUAL INSTRUCTIONS

REG_WR r_wave wmem [&0] -ww

REG_WR r_wave wmem [&0] -ww

R_wave and wmem should keep equal. But the cycle to read is not stalled.

Clocks:

The block has four clocks.

- ps_clk_slow (100Mhz) Clock used for AXI communication with PS

- c_clk (350 MHz) Clock used in the tCore Processor.

- t_clk (500 MHz) Clock used to run Time, clock related with the DACs

- read_clk  Clock used to read data, clock related with the ADCs

## Additional Info to be added and taken into account

- When Configuring the FPGA all the OUTS goes to 1 (around 55 ms)

- In this version Address can only came from a DREG

# Signal Generator

The 128-bit WaveParam output are connected to the Signal Generator (SG). The SG generates a waveform from 4 posible sources. Table, DDS, Product (Table * DDS) and Zero-value.

The Table is a custom waveform with an arbitrary shape. The DDS is a complex cosine/sine generator block, whose frequency can be configured using the provided interface.

The table is loaded using the AXI-Stream interface with a support DMA block.

User can specify the address of the first sample using the corresponding axi register,

to allow uploading several waveforms into the internal memory.

The DDS section is an integrated IP that works in "streaming" mode, which means frequency can be changed from sample to sample.

This allows the user to specify a precise duration for waveforms.

The number of samples of the output waveform is specified in the input interface.

This configuration interface allows to push waveforms into the internal queue (FIFO). Whenever the FIFO is empty,

the block will output zero-valued samples. When the FIFO is not empty, the block will act

accordingly to generate the waveform at the output m_axis interface.

| Operation | Description | Example |
| --- | --- | --- |
| ADD | Register plus Immediate | -op(r1 + #7) |
|  | Register Plus Register | -op(s1 + r2) |
| SUB | Register minus Immediate | -op(r1 - #7) |
|  | Register minus Register | -op(s1 - r2) |
| AND | Arithmetic Shift Right Immediate Amount | -op(r1 ASR #7) |
|  | Arithmetic Shift Right registered Amount | -op(s1 ASR r2) |
| ASR | Arithmetic Shift Right Immediate Amount | -op(r1 ASR #7) |
|  | Arithmetic Shift Right registered Amount | -op(s1 ASR r2) |

The SG generates a waveform from 4 possible sources. Table, DDS, Product (Table * DDS) and Zero-value. The Table is a custom waveform with an arbitrary shape. The DDS is a complex cosine/sine generator block, whose frequency is configured with the WaveParam.

Figure 7: Signal Generator Block Diagram

| Command | Conditional Execution | Second Data Task | Second Wave Task | Second Port Task | TEST Task |
| --- | --- | --- | --- | --- | --- |
| NOP |  |  |  |  |  |
| TEST |  |  |  |  | YES |
| REG_WR | YES |  |  |  | YES |
| DPORT_RD |  |  |  |  |  |
| DMEM_WR | YES | YES |  |  | YES |
| DPORT_WR | YES | YES | YES |  | YES |
| REG_WR |  | *YES | YES | YES | YES |
| WMEM_WR |  | YES | YES | YES | YES |
| WPORT_WR |  | YES | YES |  |  |
| JUMP | YES | YES |  | YES | YES |
| CALL |  | YES |  |  | YES |
| RET |  | YES |  |  | YES |
| TIME |  |  |  |  |  |
| ARITH |  |  |  |  |  |
| DIV |  |  |  |  |  |
| COND |  |  |  |  |  |