Topic Area: 11 Licensing, Safety, and Security

PI: TBD:

Collaborators: Andrew Hahn (Sandia, ARCADE), Alex Edwards (UT-Grad-Student)

Overview

The cybersecurity of remotely operated advanced reactors will become critical part of design for future reactors relying on remote or autonomous/semi-autonomous operation. Exploring the challenges presented by developing highly complex cyber-nuclear systems must be an integral part of research to ensure the systems developed are safe and secure from bad actors as well as accidents.

This proposal will both map and address possible cyber-attack vectors specific to both nuclear systems as well as general cyber-physical ICS systems. This will be accomplished both through creating non-physical models and demonstrating security architecture on an experiment in the NETL-TRIGA UT reactor. The non-physical models will be developed using the newly developed Sandia nuclear-cyber code ARCADE to map out possible attack vectors and supplemented by the Sandia SCEPTER code. The physical demonstration will be accomplished by implementing a digital twin verification model into a real remote operation system on a power effecting experiment installed into the UT NETL TRIGA reactor to emulate a remotely controlled nuclear system.

A) Identifying and Mapping Critical Attack Vectors Using Non-Physical Modeling (ARCADE)

To identify vulnerabilities in the system being evaluated, in this case the remotely controlled device to be installed in the UT NETL reactor. The Sandia code ARCADE (Advanced Reactor Cyber Analysis and Development Environment) will be used to simulate the existing reactor and its PLC (programmable logic controllers) and console endpoints. ARCADE may also be expanded to include TCA level 2 (network analysis) of the NRCs Cyber design methodology in order to include the remote operation infrastructure that will be implemented in the physical model. This will result in the following:

- TCA Level 1 Model in ARCADE – including a simulated physical model of the NETL reactor, emulated endpoints for every control surface in the model (there will only be one endpoint on the reactor for this simple non-physical model)

- Digital Twin Development - Digital twins for the Digital Twin Certification System (DTCS) will be developed and implemented on the emulated endpoints as a demonstration of security architecture

- TCA Level 2 Model integrated with ARCADE – includes basic networking infrastructure emulation to model a remotely controlled power effecting device.

- Basic Penetration Testing – Demonstration of some possible attack vectors using vulnerabilities detected during the development of the model and methods for mitigation of these attacks and other similar attacks

- (additional) Development of similar SCEPTER Model – SCEPTER is a similar code. It may be useful to develop an additional model using SCEPTER

The development of this non-physical model will have multiple benefits. The development will inform design decisions made for the demonstration as well as aid in the development of ARCADE itself. ARCADE is a new code being built to address the NRCs new cyber-nuclear methodology and has not been implemented to emulate a real nuclear system.

B) Physical Model Demonstration on UT NETL TRIGA Reactor

The physical model will implement the architecture modeled by the non-physical ARCADE implementation. This will result in the following:

- Power Smoothing Rod – An experiment will be installed in the NETL reactor that has limited control over reactivity. This will serve as the control surface the remote system has access to

- DTCS system implementation – Two servers on each side of the remote system will be stood up to run reduced order digital twins used to verify data and commands traveling to and from the system.

- Reactor Sensor Outputs – The reactors sensor outputs will be given to the DTCS system

- Remote Operator Terminal – A remote terminal will be developed to give commands verified by the DTCS system before implementing commands

- ARCADE Verification – The physical model will be compared with the developed non-physical ARCADE model to find areas for improvement in both ARCADE itself and the physical model.

Collaboration & Roles

- University of Texas (Lead): Primary technical execution, integration on UT TRIGA, planning/control development (MPC), interfaces, operator workflow design, testing, and demonstrations.

- Sandia National Lab (Guidance/Oversight): Technical oversight and review, especially around development of the ARCADE model