DOE NEUP FY 2026 CINR Pre-Application (DE-FOA-0003538) – Technical Narrative

Topic Area 11 – Licensing, Safety, and Security

Digital Twin–Enabled Materials Control and Accountability for Molten Salt Reactors

PI: William Charlton, University of Texas at Austin

Subrecipients: Karen K. Hogue ORNL, Ryan Stewart, INL

Period: 36 months

DOE Request: $1,000,000

# 1. Problem Statement, and Mission Relevance

Liquid-fueled molten salt reactors (MSRs) present a licensing and safeguards challenge because special nuclear material (SNM) is present in a bulk form that circulates through reactor systems and is transported and transmuted through coupled chemical and physical processes [1]. Fissile material moves through process lines, drain tanks, off-gas systems, and other pathways, potentially depositing as holdup through solidification, corrosion, plating, and other material interactions. These characteristics complicate traditional materials control and accountability (MC&A) methods based on discrete-item tracking, creating regulatory uncertainty in timely detection and credible understanding of theft and loss pathways.

To address this challenge, the proposed work will experimentally demonstrate a digital twin-enabled MC&A approach that uses conventional MSR instrumentation and rigorous uncertainty quantification (UQ) to infer fissile inventory and detect anomalies indicative of material loss, theft, or holdup. The resulting effort will establish a licensing-relevant technical basis for digital-twin MC&A, including quantified detection thresholds, timelines, and controlled false-positive rates. By leveraging existing facility sensors and software-based inference, the approach also targets economic benefits for regulation and deployment through reduced reliance on specialized in-situ detection systems.

Building on ongoing research under the Texas Digital Twin Nuclear Reactor Program and the Digital Molten Salt Reactor Initiative funded by the State of Texas, this work directly addresses NEUP Topic Area 11 by developing an innovative digital-twin-enabled MC&A methodology that enhances domestic safeguards, addresses gaps in licensing technical requirements for advanced reactors, and reduces regulatory uncertainty for their safe and secure operation.

# 2. State of the Art and Technical Innovation

The Nuclear Regulatory Commission (NRC) is the primary agency responsible for establishing and enforcing regulations for domestic safeguards of advanced reactors, including MSRs. Because MSRs are an emergent technology, MSR safeguards continue to evolve using concepts developed at U.S. national laboratories and universities. Many of these nascent MSR safeguards concepts rely on immature in-situ technologies or sampling strategies to assay salt isotopics and determine fissile concentration. These specialized detection technologies often require invasive key measurement points (KMPs), which are rarely considered during early reactor design phases. While valuable, these safeguards technologies are currently underdeveloped and face practical hurdles in high-temperature, high-radiation, corrosive molten salt environments.

The minimally invasive and broadly applicable solution for MC&A is to use as-built MSR plant instrumentation to infer fissile inventory, thereby avoiding reliance on unique instruments and accommodating any MSR concept throughout its design revisions. In this paradigm, fissile material accountability is achieved using real-time digital twins composed of reduced order models (ROMs) and surrogate models trained from high-fidelity neutronics, thermal-hydraulics, and chemistry simulations and calibrated to operational data.

Figure 1 – The real-time digital twin display for the UT TRIGA.  The upper left plot shows DT predicted power and measured reactor power, the middle-left plot shows the measured control rod positions, and the lower left plot shows the measured temperatures used by the digital twin.  The upper right plot shows the computed thermal flux distribution in the core, and the lower right plot shows the flux spectrum at a location near the regulating control-rod.

Our proposed innovation is the experimental demonstration of a digital twin-enabled MC&A methodology that uses an ensemble of conventional sensors (e.g., ex-core neutron detectors, thermocouples, flow measurements, and other plant signals) together with high-fidelity MSR multiphysics simulations to produce reduced-order, real-time inference models with formal uncertainty quantification (UQ). UT Austin has developed and deployed digital twins for reactor and molten salt systems [2–4] and has created MSR simulation tools needed to generate high-quality datasets for MSR digital twins and to represent MSR phenomena, including coupled neutronics and thermal-hydraulics with delayed neutron precursor drift [5–7]. ORNL leads the development of domestic safeguards approaches and technologies for MSRs [8–10], while INL advances digital twin architectures for data trustworthiness, systems integration, and anomaly detection of reactor systems [11–13]. Together, our efforts will enable the first experimental demonstration of a licensing-relevant technical basis for MC&A that integrates high-fidelity MSR physics simulations, conventional MSR instrumentation, and uncertainty-aware digital twins to provide an innovative safeguards capability across a broad range of MSR designs.

# 3. Technical Approach

A digital twin (DT) is a digital representation of a physical system in which data flow bidirectionally between the physical and digital twins, with both integrating the available data in real time. Under the Texas Digital Twin Nuclear Reactor Program and the Digital Molten Salt Reactor Initiative, The University of Texas at Austin has developed (and continues to advance) real-time digital twins for the UT TRIGA research reactor [2–4], two thermal convection molten salt loops [14–17], and a pumped salt irradiation loop [18]. Figure 1 illustrates the real-time digital twin display for the UT TRIGA reactor. This DT continuously ingests reactor instrumentation and control (I&C) signals and predicts power and neutron flux distributions and spectra at arbitrary locations within the core using a dynamic mode decomposition (DMD) ROM trained on high-fidelity neutronics simulations coupled with a point-kinetics model. The DT can also generate faster-than-real-time predictions, such as recommended control rod movements, based on the current reactor state and an objective. UT has also developed similar ROM-based DTs using alternative techniques, including proper orthogonal decomposition (POD) and long short-term memory (LSTM) models.

Figure 2 – Steady-state graphite and fuel salt temperature distribution results of a coupled neutronics thermal-hydraulics simulation using the UT-Austin developed MSRE progression problems [5,6].

During calibration of the UT TRIGA digital twin, it became evident that prediction of core reactivity depends on the accurate representation of local fuel composition due to burnup and control rod composition due to boron depletion, particularly during transients associated with startup, power maneuvers, and shutdown. Transient behavior is driven by neutronic kinetics parameters, including delayed neutron group fractions (βᵢ), which depend on the spatial isotopic distribution of delayed neutron precursors generated by different fissile isotopes. As a result, the effective delayed neutron fraction (βeff) differs substantially between a fresh uranium-fueled core and a burned core with increased plutonium content. Accurate knowledge of the local fissile inventory is therefore essential for a DT to correctly predict both current and future reactor behavior. The key insight motivating our proposed MC&A approach is to invert this relationship: if a previously calibrated digital twin using an ensemble of sensors systematically or abruptly fails to predict observed reactor behavior, the discrepancy provides evidence of a change in fissile inventory consistent with material loss, theft, or holdup in a circulating-fuel system.

Under the ongoing Digital Molten Salt Reactor Initiative and the Natura Resources Research Alliance, UT has developed advanced modeling and simulation capabilities to represent the coupled neutronic and thermal-hydraulic behavior of molten salt reactors, including systems derived from the Molten Salt Reactor Experiment (MSRE) and the Molten Salt Research Reactor (MSRR). Figure 2 illustrates results from one such simulation, showing predicted steady-state temperature distributions in the flowing molten salt and the graphite moderator. In the absence of a physical MSR digital twin, current efforts focus on developing fast-running, engineering-scale codes and ROMs that reproduce reactor behavior with accuracy approaching that of high-fidelity simulations. These models are systematically benchmarked against high-fidelity results using a structured set of progression problems of increasing complexity. Through this process, MSR digital twins are being developed in which high-fidelity simulations serve as a surrogate for instrumentation and control (I&C) data from a physical system, while fast codes and ROMs provide real-time state estimation and prediction. This digital-twin paradigm naturally extends beyond the reactor core to encompass the full MSR facility, including process piping, drain tanks, off-gas systems, and other material transport pathways that are necessary to correctly account for in-process fissile material. Although no single conventional sensor can measure fissile inventory directly, an ensemble of conventional signals can provide statistically detectable signatures of fissile inventory changes when interpreted through an appropriately calibrated digital twin.

# 4. Scope of Work

This project will build on prior work to develop a fast-running, multiphysics-based MSR digital twin capable of detecting MC&A-relevant anomalies, including losses of fissile inventory. Because operational MSRs are not currently available for full-scale data generation, MSR “ground truth” will be established by generating data from high-fidelity multiphysics simulations. Surrogate models and reduced-order models (ROMs) will simulate real-time monitoring of the MSR. Pathway-specific models will represent:

Off-gas transport and processing

Drain tank transfers and salt redistribution during operations/maintenance.

Sampling operations and declared/undeclared perturbations to salt handling.

Corrosion/plating and structural deposition

Protracted (slow) removal vs. abrupt (step) removal scenarios to bound timelines.

These pathway models enable sensitivity and observability analyses that connect mechanisms to sensor signatures and uncertainty drivers, forming the technical basis for licensing and MC&A plan arguments.

Reduced-order models suitable for real-time operation will be developed and calibrated against the high-fidelity truth data. Methods will include physics-informed surrogate modeling and dynamic mode decomposition–based reduced order models for reactor state reconstruction [4]. Measurement models will account for noise, bias, and drift in each sensor class. UQ will propagate uncertainties through the estimation and anomaly detection pipeline to produce probability of detection as a function of diverted mass and time, as well as false-alarm performance under realistic operational transients. Anomaly detection will combine multi-sensor residual testing and change-point detection, with pathway likelihood ranking when supported by evidence.

The experimental component will use the NETL TRIGA research reactor as an operational testbed. The NETL TRIGA digital twin/digital shadow workflow will be used to implement DT-MC&A inference and anomaly detection using conventional reactor signals [2,3]. A fissile-bearing irradiation sample will be inserted in-core during operations and periodically removed during planned overnight shutdowns. DT-MC&A will be evaluated on its ability to detect removal events, quantify confidence bounds and detection times, and discriminate removal signatures from normal operational variability. This demonstration provides regulator-relevant validation of the central claim: detectability of inventory changes using conventional sensors plus calibrated models.

ORNL safeguards analyses anticipate that NRC may require a formal MC&A plan for liquid-fueled MSR license applications and emphasize challenges for bulk SNM inventories [10]. DT-MC&A is structured to produce regulator-usable artifacts: documented modeling and calibration bases, validated detection performance with false-alarm control, and explicit pathway coverage arguments. Economically, DT-MC&A can reduce compliance costs by minimizing specialized instrumentation and can reduce regulatory friction by providing clearer, model-based evidence packages that improve schedule confidence for advanced reactor deployment.

The project will be executed over 36 months using three integrated thrusts with defined milestones and deliverables.

Thrust A (Months 1–18): MSR virtual testbed and pathway modeling; labeled datasets; baseline observability and sensitivity.

Thrust B (Months 6–30): Reduced-order DT-MC&A inference; calibration to truth; UQ; detection thresholds and diversion timelines.

Thrust C (Months 18–36): NETL TRIGA DT integration; fissile sample insertion/removal campaigns; detection performance; regulator-facing evidence package.

## Milestones and key deliverables

| Time | Milestone | Deliverable(s) |
| --- | --- | --- |
| M6 | Pathway definitions + initial MSR truth model | Pathway model set; baseline MSR datasets; observability assessment |
| M12 | Prototype DT-MC&A estimator + UQ plan | Estimator prototype; measurement models; initial detection metrics (simulation) |
| M18 | Calibrated inference + diversion sensitivity | Calibration workflow; uncertainty propagation; sensitivity curves (simulation) |
| M24 | TRIGA DT integration readiness | Integrated pipeline; test protocol; acceptance tests on historical TRIGA data |
| M30 | TRIGA campaigns complete | Pd vs Pfa for step/repeated removals; lessons learned |
| M36 | Regulator-facing evidence package | Final DT-MC&A framework; documentation; NRC-oriented templates/recommendations |

Feasibility is supported by UT Austin’s demonstrated TRIGA digital twin workflow [2,3] and established MSR modeling/verification capabilities [5–7]. ORNL provides MSR heritage grounding through MSRE documentation and liquid-fueled MC&A analyses [8–10]. INL contributes digital twin architectures for remote monitoring and resilient operations that inform data trustworthiness and systems integration [11–13].

# 6. Expected Outcomes and Impact

Deliverables include: (1) DT-MC&A software workflow for inventory-proxy inference and anomaly detection using conventional sensors; (2) quantified detection thresholds, false-alarm performance, and diversion timelines for representative pathways with uncertainty bounds; (3) an operating-reactor demonstration on NETL TRIGA using fissile sample insertion/removal scenarios; and (4) regulator-facing artifacts documenting evidence requirements, calibration/UQ practices, and pathway coverage arguments suitable for MSR MC&A plans.

# References

[1] U.S. Nuclear Regulatory Commission, “Digital twins for nuclear safeguards and security: Assessment of challenges, opportunities, and current state-of-practice,” Technical Letter Report TLR-RES/DE/REB-2023-08, Washington, DC, USA, Oct. 5, 2023, ADAMS Accession No. ML23271A055. https://www.nrc.gov/docs/ML2327/ML23271A055.pdf

[2] N. P. Luciano, J. R. Ross, B. Collins, W. Charlton, and K. T. Clarno, “Developing a digital twin of the TRIGA II research reactor at The University of Texas at Austin,” Transactions of the American Nuclear Society, 2025. https://www.ans.org/pubs/transactions/article-58488

[3] J. R. Ross, Development of a Digital Twin for Online State Reconstruction in TRIGA Reactors, Ph.D. dissertation, The University of Texas at Austin, Austin, TX, USA, Dec. 2024. https://doi.org/10.26153/tsw/59600

[4] J. R. Ross, “A multiphysics reduced order model in TRIGA reactors using dynamic mode decomposition,” 2025. https://www.ans.org/pubs/proceedings/article-58116

[5] S. M. Park, C. A. Gentry, N. P. Luciano, K. T. Clarno, et al., “A multiscale 3-D/1-D multiphysics molten salt reactor simulation framework based on Moltres–SAM coupling,” Proceedings of PHYSOR 2026, in preparation.

[6] C. A. Gentry, R. Krpan, K. T. Clarno, et al., “Graphite-moderated molten salt reactor progression problems,” Proceedings of M&C 2025, 2025. https://www.ans.org/pubs/proceedings/article-58239

[7] C. Miller, B. Collins, and K. Clarno, “A convergence study of the SyTH thermal fluid flow system for time dependent problems,” Proceedings of the ANS Winter Meeting, 2025 https://www.ans.org/pubs/transactions/article-59950

[8] K. K. Hogue, N. Luciano, M. Krupcale, R. Elzohery, and L. G. Evans, Planning for Material Control and Accounting at Liquid-Fueled Molten Salt Reactors, ORNL/SPR-2023/3181, Oak Ridge National Laboratory, Oak Ridge, TN, USA, Jan. 2024. https://www.sandia.gov/app/uploads/sites/273/2024/01/MSR_MCA_Approach_Jan24_final.pdf

[9] M. D. Shah, K. K. Hogue, N. C. Rowe, and S. S. Chirayath, Material Control and Accounting for Liquid-Fueled Molten Salt Reactors: Material Control and Holdup Considerations, ORNL/SPR-2024/3555, Oak Ridge National Laboratory, Oak Ridge, TN, USA, Sept. 2024. https://www.sandia.gov/app/uploads/sites/273/2024/11/Material-Control-and-Accounting-for-Liquid-Fueled-Molten-Salt-Reactors-Material-Control-and-Holdup-1.pdf

[10] S. E. Skutnik, P. W. Sobel, M. W. Swinney, K. K. Hogue, M. M. Arno, and S. S. Chirayath, “Survey of prospective techniques for molten salt reactor feed monitoring,” Annals of Nuclear Energy, vol. 208, p. 110796, 2024, doi: 10.1016/j.anucene.2024.110796.

[11] N. Hergesheimer, E. Ryan, K. Heaps, J. Cavaluzzi, A. Shields, L. Browning, C. Ritter, and R. Stewart, “Developing a digital twin framework for remotely monitoring nuclear reactor facilities,” Nuclear Engineering and Design, vol. 446, p. 114585, 2026, doi: 10.1016/j.nucengdes.2025.114585.

[12] R. Stewart, A. Shields, S. Bays, C. Ritter, G. Reyes, and M. Schanfein, “Utilizing digital twins for nuclear safeguards and security,” Proceedings of the Institute of Nuclear Materials Management Annual Meeting, Mar. 2023. https://resources.inmm.org/sites/default/files/2023-07/finalpaper_210_0513113916.pdf

[13] R. Stewart, E. Ryan, and A. Shields, AGN-201 Digital Twin Concept of Operations, INL/RPT-24-80534, Idaho National Laboratory, Idaho Falls, ID, USA, Oct. 2024. https://inldigitallibrary.inl.gov/sites/sti/sti/Sort_133344.pdf

[14] E. Rozak, E. B. Issa, C. Gentry, and K. Clarno, “Modeling natural circulation of single-phase molten salt,” Transactions of the American Nuclear Society, 2024. https://www.ans.org/pubs/proceedings/article-58047

[15] T. Villarreal, E. Rozak, K. Clarno, and Y. Hassan, “An integrated operation and simulation approach for design and analysis of molten salt-based systems,” Transactions of the American Nuclear Society, 2024. https://www.ans.org/pubs/transactions/article-57133

[16] A. F. Lopez, G. Giudicelli, M. T. Retamales, E. Rozak, and K. Clarno, “Validation of Pronghorn for molten salt natural-circulation loops,” Transactions of the American Nuclear Society, 2025. https://www.ans.org/pubs/transactions/article-59744

[17] N. Luciano, T. Head, L. Carasik, C. Gentry, and K. Clarno, “Molten salt thermal convection loops for generating high-fidelity data for digital twins,” Transactions of the American Nuclear Society, 2026, in preparation.

[18] S. D. Dayawansa, Integrated Modeling to Design a Digital Twin of the Massachusetts Institute of Technology Molten Salt Irradiation Flow Loop, M.S. thesis, The University of Texas at Austin, Austin, TX, USA, Aug. 2024. https://doi.org/10.26153/tsw/56830